# 9PK- SYSTEM

# GRANULAR/PELLETIZER

# OPERATION BOOK

## SHENYANG C&A MACHINERY

The machine is one of the fodder machine, the final products is suitable for all the animal. The water moisture is less than 10%,and the fodder could be dry or wet.

This system machines are also suitable for making the oil cake for the feedstuff, which it could be granule /pellet for the fish,chicken,animal,etc.

In Africa more are used for making the cassava pellet for the food . Also it could be used for making the biomass, sawdust pellet, etc.

Now it is adjusted to be used for making the pellet for the fuel from the sawdust,waste chips ,etc.

It has been installed with the electrical control box.

The final size is Dia 2mm, 4mm, 6mm, 8mm, 10mm,12mm ,etc.

- Usement

It is suitable for making the granule, which the final products could prompt the animal appetite, and reduce the labor of feeder.

- Characteristic

The final product from the machine, which the shape is smooth, moderatel hardness, the inter is heated moderately to make starch alke paste  and make protein coagulate, so it could prompt the  animal alimentation and assimilation ,also get the more profits.

- Installment and operation

1. Installing the machine in one line and keep its firmly on the ground floor . Then contact with the power.

2. Loose the screw for pressing the rollers, make the machine is without pressure, only it runs better ,then normally feed the raw materials.

3. Firstly using the new machine, with the 5kg orso cake flour or wheat flour with 1.5 kg vegetable oil or waste machinery oil for mixing together . Tighten the screw to make the space of roller and mould is 0.1 – 0.3 mm ,slowly feed the raw materials mixed, wait the pellet out ,then add the small sand 3-5kg to extrude 2-3 times ,to make the hole of mould is smooth.

Then slowly add into the raw materials more. If the resistance is bigger or not feeding the materials, to polish the hole of mould again .

4. when produce the feedstuff ,if more fine fibre ,pls add into 5% water ,the water would be steamed off when extruding .

5. After finishing producing ,to loose the adjusting-screw ,let the rolls freely. Stop the machine to clear the residue materials in the hopper, check the wore of rolls and mould ,order to adjust the space between the rolls and mould next using .

6. check the screws ,especially note the screw nut under the machines, using for some time to adjust up it ,to guantee the contacting of gears better . Regarding to the machines with gearbox, check the oil place is 1/2 for the machinery oil in the gearbox. The bearings should be washed and lubricate per 3 months, the bearings in the rollers should be lubricated per half month (it is better for the high temperature lubricate oil) .

Note to check all parts, including to the belts, belts, directiong of running of motor. Make the parallel template (in the hopper with many holes) balance (adjust it in 0.1-0.3mm) ,then get 10kg flour mixing 1kg food oil ,put into hopper ,then open the motor to press the flour for more times. At the beginning of new machine pressing ,carefully watch the hole whether all producing the granule,because it directly effect the rate of producing . Also keep the hole and template smooth.

Size of the granule, is confirmed by the adjusting knife, which is up of the outer ,you could adjust it up and down to get the different size of granule,also it’s confirmed by the template .

7. maintainance

| failure | reason | Salvation |
| --- | --- | --- |
| 1 when open the machine,the hole is jagged ,not come any pellet | 1.the hole is jamed 2.the hols is rust,not smooth 3.the space of mould and rollers is bigger | - clear the hole - use the high oil materials to extrude for several times - adjust its space to 0.1 -0.3 mm |
| 2.low powder in the pellet ,or the pellet is not shape | 1.the materials is low powder 2.high moistuer in the materials 3.high crude fibre in the materials | 1.improve the moisture 2.reduce the moisture 3.add the glutinous powder |
| 3.the capacity is slowly small | 1. big space between the rollers and mould 2.high moisture | 1. adjust the space 2.add more moisture |
| 4.the pellet become hard and ashine | Low moisture in the powder | Add the moisture of powder |
| 5.pellet become softy and the surface craze and roughness | 1.not suitable moisture 2.new mould | 1.adjust the moisture 2.extrude the mould with oil materials |
| 6.The machine is suddenly stop | Big hard material or metal go into the space between the rollers and mould | Clear the eyewinker |

- Safety

Prevent the operator make its hand into hopper, which could use the stick to mixing in the hopper. Note to let any iron into hopper.

If the machine couldn’t meet its capacity, after working for some time the template is smooth, it could come to the normal level.

- Dada

| Type | Power | Speed of shaft | Capacity | Outer dimension |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 9PK-105 | 0.75-1.1 | 400 rpm | 30-50 |  |  |  |
| 9PK-120 | 1.5 | 400 rpm | 40-60 |  |  |  |
| 9PK-150 | 1.5-5.5 kw | 400rpm | 50-100kg/h | 410*420*700mm |  |  |
|  | moisture of granule | Size of granule | Net weight |  |  |  |
|  | Less than 8% | ф4-12mm | 100 kg |  |  |  |
| 9PK-205 | 3-5.5 kw | 400rpm | 100-200kg/h | 810*360*800mm |  |  |
|  | moisture of granule | Size of granule | Net weight |  |  |  |
|  | Less than 8% | ф4-12mm | 160 kg |  |  |  |
| 9PK-200 | 5.5-7.5 kw | 450rpm | 150-200kg/h |  | 1100*520*1500mm |  |
|  | moisture of granule | Size of granule | Net weight |  |  |  |
|  | Less than 8% | ф4-12mm | 240 kg |  |  |  |
| 9PK-250 | 5.5-7.5 kw | 400rpm | 400-800kg/h |  |  |  |
|  | moisture of granule | Size of granule | Net weight |  |  |  |
|  | Less than 8% | ф4-12mm | 280 kg |  |  |  |
| 9PK-300 | 7.5 -11kw | 400rpm | 600-1000kg/h |  |  |  |
|  | moisture of granule | Size of granule | Net weight |  |  |  |
|  | Less than 8% | ф4-12mm | 460 kg |  |  |  |
| 9PK-350 | 11 -15kw | 500rpm | 900-1200kg/h |  |  |  |
|  | moisture of granule | Size of granule | Net weight |  |  |  |
|  | Less than 8% | ф4-12mm | 560 kg |  |  |  |

- Note: the capacity only is for sawdust pellet, if for feedstuff the capacity is double.

- Grantee time:3 months except for spare parts.

- Warning!

The motor is more than 18.5kw, the user should match with the electrical high voltage safeguard parts before using, or it would destroy the motor.

Note: Supply for the following line .

- Feedstuff pellet line for animal

Oil cake – Cake Grinder- crushed powder- Granular – cake pellet

- Wood/sawdust pellet line for the fuel

Wood/sawdust–wood Grinder-wood Granular – cool-wood pellet

Best quality, satisfying service!